6502 Instruction Summary

Originally by RTK, on 23-Jul-97

Redone by CricketNE, last update: 01-March-2000

Please read teh README.TXT taht comes with this archive

as there is some important info in it.

Summary format:

--------------

Mnemonic  -  Instruction name

Operation -  Processor status flags affected

Just as a quick mental note:

# means immediate addressing

$ means a hexadecimal value

() means some form of indirect addressing

Lets begin shall we?

======================================================================

ADC  -  Add to Accumulator with Carry

Flags Affected:

---------------

A+M+C -> A,C                              N, Z, C, V

Mnemonic   Addressing Mode       Opcode

---------------------------------------

ADC        #aa                   69

ADC        aa                    65

ADC        aa,X                  75

ADC        aaaa                  6D

ADC        aaaa,X                7D

======================================================================

AND  -  AND Memory with Accumulator

Flags Affected:

---------------

A and M -> A                              N, Z

Mnemonic   Addressing Mode       Opcode

---------------------------------------

AND        #aa                   29

AND        aa                    25

AND        aa,X                  35

AND        aaaa                  2D

AND        aaaa,X                3D

AND        aaaa,Y                39

AND        (aa,X)                21

AND        (aa),Y                31

======================================================================

ASL  -  Arithmatic Shift Left

Flags Affected:

---------------

+-+-+-+-+-+-+-+-+                	N, C, Z

C <- |7|6|5|4|3|2|1|0| <- 0

+-+-+-+-+-+-+-+-+

Mnemonic   Addressing Mode       Opcode

---------------------------------------

ASL        A                     0A

ASL        aa                    06

ASL        aa,X                  16

ASL        aaaa                  0E

ASL        aaaa,X                1E

======================================================================

BCC  -  Branch on Carry Clear

Flags Affected:

---------------

Branch On C=0                             No Flags

Mnemonic   Addressing Mode       Opcode

---------------------------------------

BCC        aa                    90

======================================================================

BCS  -  Branch on Carry Set

Flags Affected:

---------------

Branch On C=1                             No Flags

Mnemonic   Addressing Mode       Opcode

---------------------------------------

BCS        aa                    B0

======================================================================

BEQ  -  Branch Zero Set

Flags Affected:

---------------

Branch On Z=1                             No Flags

Mnemonic   Addressing Mode       Opcode

---------------------------------------

BEQ        aa                    F0

======================================================================

BIT  -  Test Bits in Memory with Accumulator

A and M, M7 -> N, M6 -> V          N=M7, V=M6, Z=1 if A and M = 0

neither A nor M are altered

Mnemonic   Addressing Mode       Opcode

---------------------------------------

BIT        aa                    24

BIT        aaaa                  2C

======================================================================

BMI  -  Branch on Result Minus

Flags Affected:

---------------

Branch On N=1                             No Flags

Mnemonic   Addressing Mode       Opcode

---------------------------------------

BMI        aa                    30

======================================================================

BNE  -  Branch on Z reset

Flags Affected:

---------------

Branch On Z=0                             No Flags

Mnemonic   Addressing Mode       Opcode

---------------------------------------

BNE        aa                    D0

======================================================================

BPL  -  Branch on Result Plus (or Positive)

Flags Affected:

---------------

Branch On N=0                             No Flags

Mnemonic   Addressing Mode       Opcode

---------------------------------------

BPL        aa                    10

======================================================================

BRK  -  Force a Break

Flags Affected:

---------------

forced interrupt                          B=1, I=1

Mnemonic   Addressing Mode       Opcode

---------------------------------------

BRK                              00

======================================================================

BVC  -  Branch on Overflow Clear

Flags Affected:

---------------

Branch On V=0                             No Flags

Mnemonic   Addressing Mode       Opcode

---------------------------------------

BVC        aa                    50

======================================================================

BVS  -  Branch on Overflow Set

Flags Affected:

---------------

Branch On V=1                             No Flags

Mnemonic   Addressing Mode       Opcode

---------------------------------------

BVS        aa                    70

======================================================================

CLC  -  Clear Carry Flag

Flags Affected:

---------------

0 -> C                                    C=0

Mnemonic   Addressing Mode       Opcode

---------------------------------------

CLC                              18

======================================================================

CLD  -  Clear Decimal Mode

Flags Affected:

---------------

0 -> D                                    D=0

Mnemonic   Addressing Mode       Opcode

---------------------------------------

CLD                              D8

======================================================================

CLI  -  Clear Interrupt Disable

Flags Affected:

---------------

0 -> I                                    I=0

Mnemonic   Addressing Mode       Opcode

---------------------------------------

CLI                              58

======================================================================

CLV  -  Clear Overflow Flag

Flags Affected:

---------------

0 -> V                                    V=0

Mnemonic   Addressing Mode       Opcode

---------------------------------------

CLV                              B8

======================================================================

N.B. CMP, CPX, and CPY all set flags according to:

A, X, or Y < Memory  --->  N=1, Z=0, C=0

A, X, or Y = Memory  --->  N=0, Z=1, C=1

A, X, or Y > Memory  --->  N=0, Z=0, C=1

CMP  -  Compare Memory and Accumulator

Flags Affected:

---------------

A compared with M                         N, Z, C

Mnemonic   Addressing Mode       Opcode

---------------------------------------

CMP        #aa                   C9

CMP        aa                    C5

CMP        aa,X                  D5

CMP        aaaa                  CD

CMP        aaaa,X                DD

CMP        aaaa,Y                D9

CMP        (aa,X)                C1

CMP        (aa),Y                D1

CPX  -  Compare Memory and X register

Flags Affected:

---------------

X compared with M                         N, Z, C

Mnemonic   Addressing Mode       Opcode

---------------------------------------

CPX        #aa                   E0

CPX        aa                    E4

CPX        aaaa                  EC

CPY  -  Compare Memory and Y register

Flags Affected:

---------------

Y compare with M                          N, Z, C

Mnemonic   Addressing Mode       Opcode

---------------------------------------

CPY        #aa                   C0

CPY        aa                    C4

CPY        aaaa                  CC

======================================================================

DEC  -  Decrement Memory by One

Flags Affected:

---------------

M - 1 -> M                                N, Z

Mnemonic   Addressing Mode       Opcode

---------------------------------------

DEC        aa                    C6

DEC        aa,X                  D6

DEC        aaaa                  CE

DEC        aaaa,X                DE

======================================================================

DEX  -  Decrement X

Flags Affected:

---------------

X - 1 -> X                                N, Z

Mnemonic   Addressing Mode       Opcode

---------------------------------------

DEX                              CA

======================================================================

DEY  -  Decrement Y

Flags Affected:

---------------

Y - 1 -> Y                                N, Z

Mnemonic   Addressing Mode       Opcode

---------------------------------------

DEY                              88

======================================================================

EOR  -  Exclusive-OR Memory with Accumulator

Flags Affected:

---------------

A xor M -> A                              N, Z

Mnemonic   Addressing Mode       Opcode

---------------------------------------

EOR        #aa                   49

EOR        $aa                   45

EOR        $aa,X                 55

EOR        $aaaa                 4D

EOR        $aaaa,X               5D

EOR        $aaaa,Y               59

EOR        ($aa,X)               41

EOR        ($aa),Y               51

======================================================================

INC  -  Increment Memory by one

Flags Affected:

---------------

M + 1 -> M                                N, Z

Mnemonic   Addressing Mode       Opcode

---------------------------------------

INC        $aa                    E6

INC        $aa,X                  F6

INC        $aaaa                  EE

INC        $aaaa,X                FE

======================================================================

INX  -  Increment X by one

Flags Affected:

---------------

X + 1 -> X                                N, Z

Mnemonic   Addressing Mode       Opcode

---------------------------------------

INX                              E8

======================================================================

INY  -  Increment Y by one

Flags Affected:

---------------

Y + 1 -> Y                                N, Z

Mnemonic   Addressing Mode       Opcode

---------------------------------------

INY                              C8

======================================================================

JMP  -  Jump

Flags Affected:

---------------

No Flags

Mnemonic   Addressing Mode       Opcode

---------------------------------------

JMP        $aaaa                 4C

JMP        ($aaaa)               6C

======================================================================

JSR  -  Jump to subroutine

Flags Affected:

---------------

No Flags

Mnemonic   Addressing Mode       Opcode

---------------------------------------

JSR        $aaaa                 20

======================================================================

LDA  -  Load Accumulator with memory

Flags Affected:

---------------

M -> A                                    N, Z

Mnemonic   Addressing Mode       Opcode

---------------------------------------

LDA        #aa                   A9

LDA        $aa                   A5

LDA        $aa,X                 B5

LDA        $aaaa                 AD

LDA        $aaaa,X               BD

LDA        $aaaa,Y               B9

LDA        ($aa,X)               A1

LDA        ($aa),Y               B1

======================================================================

LDX  -  Load X with Memory

Flags Affected:

---------------

M -> X                                    N, Z

Mnemonic   Addressing Mode       Opcode

---------------------------------------

LDX        #aa                   A2

LDX        $aa                   A6

LDX        $aa,Y                 B6

LDX        $aaaa                 AE

LDX        $aaaa,Y               BE

======================================================================

LDY  -  Load Y with Memory

Flags Affected:

---------------

M -> Y                                    N, Z

Mnemonic   Addressing Mode       Opcode

---------------------------------------

LDY        #aa                   A0

LDY        $aa                   A4

LDY        $aa,X                 B4

LDY        $aaaa                 AC

LDY        $aaaa,X               BC

======================================================================

LSR  -  Logical Shift Right

Flags Affected:

---------------

+-+-+-+-+-+-+-+-+ 	                  N, C, Z

0 -> |7|6|5|4|3|2|1|0| -> C

+-+-+-+-+-+-+-+-+

Mnemonic   Addressing Mode       Opcode

---------------------------------------

LSR        A                     4A

LSR        $aa                   46

LSR        $aa,X                 56

LSR        $aaaa                 4E

LSR        $aaaa,X               5E

======================================================================

NOP  -  No Operation

Flags Affected:

---------------

No Flags

Mnemonic   Addressing Mode       Opcode

---------------------------------------

NOP                              EA

======================================================================

ORA  -  OR Memory with Accumulator

Flags Affected:

---------------

A or M -> A                               N, Z

Mnemonic   Addressing Mode       Opcode

---------------------------------------

ORA        #aa                   09

ORA        $aa                   05

ORA        $aa,X                 15

ORA        $aaaa                 0D

ORA        $aaaa,X               1D

ORA        $aaaa,Y               19

ORA        ($aa,X)               01

ORA        ($aa),Y               11

======================================================================

PHA  -  Push Accumulator on Stack

Flags Affected:

---------------

A -> Stack                                No Flags

Mnemonic   Addressing Mode       Opcode

---------------------------------------

PHA                              48

======================================================================

PHP  -  Push Processor Status on Stack

Flags Affected:

---------------

P -> Stack

Mnemonic   Addressing Mode       Opcode

---------------------------------------

PHP                              08

======================================================================

PLA  -  Pull Accumulator from Stack

Flags Affected:

---------------

Stack -> A                                N, Z

Mnemonic   Addressing Mode       Opcode

---------------------------------------

PLA                              68

======================================================================

PLP  -  Pull Processor Status from Stack

Flags Affected:

---------------

Stack -> P *** WTF? from stack ***

Mnemonic   Addressing Mode       Opcode

---------------------------------------

PLP                              28

======================================================================

ROL  -  Rotate Left

Flags Affected:

---------------

+-+-+-+-+-+-+-+-+                      N, Z, C

C <- |7|6|5|4|3|2|1|0| <- C

+-+-+-+-+-+-+-+-+

Mnemonic   Addressing Mode       Opcode

---------------------------------------

ROL        A                     2A

ROL        $aa                   26

ROL        $aa,X                 36

ROL        $aaaa                 2E

ROL        $aaaa,X               3E

======================================================================

ROR  -  Rotate Right

Flags Affected:

---------------

+-+-+-+-+-+-+-+-+                      N, Z, C

C -> |7|6|5|4|3|2|1|0| -> C

+-+-+-+-+-+-+-+-+

Mnemonic   Addressing Mode       Opcode

---------------------------------------

ROR        A                     6A

ROR        $aa                   66

ROR        $aa,X                 76

ROR        $aaaa                 6E

ROR        $aaaa,X               7E

======================================================================

RTI  -  Return from Interrupt

Flags Affected:

---------------

No Flags

Mnemonic   Addressing Mode       Opcode

---------------------------------------

RTI                              40

======================================================================

RTS  -  Return from Subroutine

Flags Affected:

---------------

No Flags

Mnemonic   Addressing Mode       Opcode

---------------------------------------

RTS                              60

======================================================================

SBC  -  Subtract from Accumulator with Carry

Flags Affected:

---------------

N, Z, C, V

A - M - ~C -> A  (~C is NOT C)

Mnemonic   Addressing Mode       Opcode

---------------------------------------

SBC        #aa                   E9

SBC        $aa                   E5

SBC        $aa,X                 F5

SBC        $aaaa                 ED

SBC        $aaaa,X               FD

SBC        $aaaa,Y               F9

SBC        ($aa,X)               E1

SBC        ($aa),Y               F1

======================================================================

SEC  -  Set Carry Flag

Flags Affected:

---------------

Mnemonic   Addressing Mode       Opcode

---------------------------------------

SEC                              38

======================================================================

SED  -  Set Decimal Mode

Flags Affected:

---------------

Mnemonic   Addressing Mode       Opcode

---------------------------------------

SED                              F8

======================================================================

SEI  -  Set Interrupt Disable

Flags Affected:

---------------

Mnemonic   Addressing Mode       Opcode

---------------------------------------

SEI                              78

======================================================================

STA  -  Store Accumulator in Memory

Flags Affected:

---------------

Mnemonic   Addressing Mode       Opcode

---------------------------------------

STA        $aa                   85

STA        $aa,X                 95

STA        $aaaa                 8D

STA        $aaaa,X               9D

STA        $aaaa,Y               99

STA        ($aa,X)               81

STA        ($aa),Y               91

======================================================================

STX  -  Store X in Memory

Flags Affected:

---------------

Mnemonic   Addressing Mode       Opcode

---------------------------------------

STX        $aa                   86

STX        $aa,Y                 96

STX        $aaaa                 8E

======================================================================

STY  -  Store Y in Memory

Flags Affected:

---------------

Mnemonic   Addressing Mode       Opcode

---------------------------------------

STY        $aa                   84

STY        $aa,X                 94

STY        $aaaa                 8C

======================================================================

TAX  -  Transfer Accumulator to X

Flags Affected:

---------------

A -> X                                    N, Z

Mnemonic   Addressing Mode       Opcode

---------------------------------------

TAX                              AA

======================================================================

TAY  -  Transfer Accumulator to Y

Flags Affected:

---------------

A -> Y                                    N, Z

Mnemonic   Addressing Mode       Opcode

---------------------------------------

TAY                              A8

======================================================================

TSX  -  Transfer Stack to X

Flags Affected:

---------------

S -> X                                    N, Z

Mnemonic   Addressing Mode       Opcode

---------------------------------------

TSX                              BA

======================================================================

TXA  -  Transfer X to Accumulator

Flags Affected:

---------------

X -> A                                    N, Z

Mnemonic   Addressing Mode       Opcode

---------------------------------------

TXA                              8A

======================================================================

TXS  -  Transfer X to Stack

Flags Affected:

---------------

X -> S                                    N, Z

Mnemonic   Addressing Mode       Opcode

---------------------------------------

TXS                              9A

======================================================================

TYA  -  Transfer Y to Accumulator

Flags Affected:

---------------

Y -> A                                    N, Z

Mnemonic   Addressing Mode       Opcode

---------------------------------------

TYA                              98

END OF FILE